Point control wall washing lamp manual

- Graphic part：

A．Function key

B．Add keys

C．Cut keys

D．Confirmation key

E．LED Display window

A        B      C     D

Instructions: press A to cycle up to eight different functions, the digital tube is the first two representatives of the function (see table). The last two digits representing the functions. The address code. Or speed parameters. Press B or C key to modify the the parameter value. Press D to confirm

1.LED display window function control table, (all the functions are selected and then press the D key to confirm)

| number | Display | Function description |
| --- | --- | --- |
| 1 | A001 | Channel address code (001 - 512) B and C keys add and subtract address code values |
| 2 | 19ch/-8ch-42ch | Channel selection (19 channels, 8 channels, 42 channels) B, C key addition and subtraction |
| 3 | CC00 | Seven color jump, (01 - 99) The B and C keys modify the jump speed |
| 4 | Ed00 | Seven colored gradients, veins change, (01 - 99) The B and C keys modify the gradient speed |
| 5 | Er00 | Seven colors running 1, (01 - 99) B, C key to modify the pulse speed (set Id, can achieve multiple table lamp flow) |
| 6 | Po00 | Seven colors running 2, (01 - 99) B, C key to modify the pulse speed (set Id, can achieve multiple table lamp flow) |
| 7 | tr00 | Running changes (00 - 8) The B and C keys modify running patterns |
| 8 | tU00 | Running changes (00 - 99) The B and C keys modify the running speed |
| 9 | BE50 | Colorful voice running changes The B, C keys modify the sound sensitivity |
| 10 | R200 | Red color selection (000 - 255) The B and C keys modify the color value |
| 11 | G200 | Green color selection (000 - 255) The B and C keys modify the color values |
| 12 | B200 | Blue color selection (000 - 255) The B and C keys modify the color values |
| 13 | ID00 | Lamps and lanterns ID, (01 - 16) The B and C keys modify the ID value (used for racing effects, allowing multiple table lamps to flow) |
| 14 | OPRT | From the machine |

2. background LED display window function control table, (in Dmx512 mode, long press function key to enter, all the functions according to select and then press the D key to confirm)

| Number | Display | Function description |
| --- | --- | --- |
| 1 | R200 | Red current, (20 - 255) The B, C keys modify the current |
| 2 | G200 | Green current, (20 - 255) The B, C keys modify the current |
| 3 | B200 | Blue current, (20 - 255) The B, C keys modify the current |

3．DMX512    8Channel description

| Number | Display | Function description |
| --- | --- | --- |
| 1CH | Channel description | R、G、B The total dimming, linear dimming, from dark to bright |
| 2CH | Total strobe | R、G、B The total strobe is slow to fast |
| 3CH | Function selection 1 | 0--25： NO 26--35： NO 36--45： Colorful water (jump); (Id yards staggered, can achieve multiple table lamp flow, controlled by 1--6 channel) 46--55： Colorful water (gradient); (Id yards staggered, can achieve multiple table lamp flow, controlled by 1--6 channel) 56--65： Color selection (controlled by 1 channels) 66--75： Jump; be controlled by 1 channels 76--85： Gradual change in veins; (controlled by 1 channels) 86--95： Running pattern 1; (controlled by 1--6 channel) 96--105： Running pattern 2; (controlled by 1--6 channel) 。。。。。。。。 236--245：Running pattern 15; (controlled by 1--6 channel) 246--255：Sound control (controlled by 1--6 channels) |
| 4CH | Function selection 2 | 0--5： NO 6--255： Fixed 255 pattern selections; controlled by 1--6 channels |
|  | Function 3 | Colorful flow; direction control (0--127: left flowing water; 128--255: right flowing water); |
| 5CH | Function speed 1 | Function speed, from slow to fast |
| 6CH | R Dimming | R dimming, linear dimming, from dark to bright |
| 7CH | G Dimming | G dimming, linear dimming, from dark to bright |
| 8CH | B Dimming | B dimming, linear dimming, from dark to bright |

4．DMX512    19Channel description

| Number | Display | Function description |
| --- | --- | --- |
| 1CH | Channel description | R、G、B The total dimming, linear dimming, from dark to bright |
| 2CH | Total strobe | R、G、B The total strobe is slow to fast |
| 3CH | Function selection 1 | 0--25： NO 26--35： Colorful color flow water (controlled by 1--20 channel) 36--45： Colorful water (jump); (Id yards staggered, can achieve multiple table lamp flow, controlled by 1--6 channel) 46--55： Colorful water (gradient); (Id yards staggered, can achieve multiple table lamp flow, controlled by 1--6 channel) 56--65： Color selection (controlled by 1 channels) 66--75： Jump; be controlled by 1 channels 76--85： Gradual change in veins; (controlled by 1 channels) 86--95： Running pattern 1; (controlled by 1--6 channel) 96--105： Running pattern 2; (controlled by 1--6 channel) 。。。。。。。。 236--245：Running pattern 15; (controlled by 1--6 channel) 246--255：Sound control (controlled by 1--6 channels) |
| 4CH | Function selection 2 | 0--5： NO 6--255： Fixed 250 pattern choices; |
|  | Function 3 | Colorful flowing water; colorful color flow water; direction control (0--127: left flowing water; 128--255: right flowing water); |
| 5CH | Function speed 1 | Function speed, from slow to fast |
| 6CH | 1 Dimming | R、G、B Dimming |
| 7CH | 2 Dimming | R、G、B Dimming |
| 8CH | 3 Dimming | R、G、B Dimming |
| 9CH | 4 Dimming | R、G、B Dimming |
| ...... | ....... | .......... |
| 19CH | 14 Dimming | R、G、B Dimming |

4．DMX512    42Channel description

| Number | Display | Function description |
| --- | --- | --- |
| 1CH | R1 Dimming | R1 Dimming, linear dimming, from dark to bright |
| 2CH | G1 Dimming | G1 Dimming, linear dimming, from dark to bright |
| 3CH | B1 Dimming | B1 Dimming, linear dimming, from dark to bright |
| 4CH | R2 Dimming | R2 Dimming, linear dimming, from dark to bright |
| 5CH | G2 Dimming | G2 Dimming, linear dimming, from dark to bright |
| 6CH | B2 Dimming | B2 Dimming, linear dimming, from dark to bright |
| 。。。。。。。。 | 。。。。。。。。 | 。。。。。。。。。。。。。。。。。。。。。。。。。。。。。。。。。 |
| 42CH | B14 Dimming | B14 Dimming, linear dimming, from dark to bright |

5. function features:

Operation is very simple, user-friendly design, dimming, flicker, jitter phenomenon, etc., can be adapted to photography, photography, television and other requirements of strict lighting occasions