180*3W full color LED city color

User’s manual

- 1. diagrammatize ：

A．funtion

B．add

C．substract

D．confirm

E．LEDdisplay window

A   B  C  D
operation instructions：press function key A will cycle out seven different function effect，LED Top row in represent the current function（refer to function table）.LED On behalf of the top row in the function of the parameters of the address and speed. Press B or C key can amend parameter values.Press D key to confirm.

2. LED display window function league table，（select all the function keys    then press D key to confirm）

| NO. | display | Function instruction |
| --- | --- | --- |
| 1 | A001 | 3 channel address code，（001—512） B、C key is add and substract address code value |
| 2 | H001 | 7 channel address code， （001—512） B、C key is add and substract address code value |
| 3 | h001 | 13 channel address code，（001—512） B、C key is add and substract address code value |
| 4 | CC50 | seven colors saltus step change，（00—99） B、C key is amend the speed of saltus stepchanging |
| 5 | CP50 | Seven colors pulse change、gradual change，（00—99） B、C key amend the speed of pulse change、gradual change |
| 6 | DE50 | Seven colors running，（00—99） B、C key amend the speed of running |
| 7 | DEDE | Seven colors sound control change， （00—99） B、C key amend the speed of sound control |
| 8 | R255 | R select colors change， （000—255） B、C key amend the speed of sound control |
| 9 | G255 | G select colors change， （000—255） B、C key amend the speed of sound control |
| 10 | B255 | B select colors change， （000—255） B、C key amend the speed of sound control |
| 11 | n255 | strobe，（000—255）,RGB colors choose is valid for strobe B、C key can increase and reduce the speed of strobe |

3．DMX512 three channesl instruction

| NO. | function | instruction |
| --- | --- | --- |
| CH 1 | R dimming | R dimming，liner dimming，from dark to bright |
| CH 2 | G dimming | G dimming，liner dimming，from dark to bright |
| CH 3 | B dimming | B dimming，liner dimming，from dark to bright |

4．DMX512 seven channesl instruction

| NO. | function | instruction |
| --- | --- | --- |
| CH 1 | Main dimming | R、G、B、main dimming，liner dimming，from dark to bright |
| CH 2 | Main strobe | R、G、B、main strobe，from slow to fast |
| CH 3 | Function choooe | 0—50：DMX512；51—99：saltus step changing；100—150：gradual change；151—200：running change；201—250：pulse change；251—255：sound control |
| CH 4 | Speed of function | The speed of function，from slow to fast |
| CH 5 | R dimming | R dimming，liner dimming，from dark to bright |
| CH 6 | G dimming | G dimming，liner dimming，from dark to bright |
| CH 7 | B dimming | B dimming，liner dimming，from dark to bright |

5．DMX512 thirteenth channels instruction

| NO. | function | instruction |
| --- | --- | --- |
| CH 1 | Main dimming | R、G、B、main dimming，liner dimming，from dark to bright |
| CH 2 | Main strobe | R、G、B、main strobe,from slow to fast |
| CH 3 | Function choooe | 0—50：DMX512；51—99：dimming，liner dimming，from dark to bright；100—150：gradual change；151—200：running change；201—250：pulse change；251—255：sound control |
| CH 4 | Speed of function | The speed of function，from slow to fast |
| CH 5 | R1 dimming | R1 dimming，liner dimming，from dark to bright |
| CH 6 | G1 dimming | G1 dimming，liner dimming，from dark to bright |
| CH 7 | B1 dimming | B1 dimming，liner dimming，from dark to bright |
| CH 8 | R2 dimming | R2 dimming，liner dimming，from dark to bright |
| CH 9 | G2 dimming | G2 dimming，liner dimming，from dark to bright |
| CH 10 | B2 dimming | B2 dimming，liner dimming，from dark to bright |
| CH 11 | R3 dimming | R3 dimming，liner dimming，from dark to bright |
| CH 12 | G3 dimming | G3 dimming，liner dimming，from dark to bright |
| CH 13 | B3 dimming | B3 dimming，liner dimming，from dark to bright |

6．Function feature：

Easy to operate，humanity design，dimmer without flashing、chattering phenomena and so on .Can    adapt to photography, photography television stations, the occasion strict to light.